MAC Implementation - Required Corrections

1. Key Selection

Key Index = (Server_ID + Loco_ID) % 2

0 = KEY_1

1 = KEY_2

2. MAC Input

Use bytes from Message Type up to the last payload byte. Do not include BB BB, MAC or CRC.

3. Padding

Split into 16-byte blocks. Pad the last block with 00. Do not use PKCS padding.

4. AES Processing

Do not use normal AES-CBC over the complete message.

Working Key = Selected Authentication Key

IV = 16 bytes of 00

For each 16-byte block:

Cipher Block = AES-128-CBC (Working Key, Zero IV, Message Block)

Working Key = Cipher Block

The cipher output of one block becomes the AES key for the next block.

5. Final MAC

Do not take the first 4 bytes of the final AES block. Calculate CRC32 over the complete final 16-byte Cipher Block. Transmit the resulting 32-bit MAC MSB first.

6. CRC32

The CRC32 logic is same as packet crc logic.

7. Verification Test

Server ID = 50002, Loco ID = 65551

MAC Input = 18 00 23 05 47 01 00 0F 61 78 01 18 09 1A 0F 05 08 03 00 10 6A 00 10 68 00 10 6C

Selected key index = 1

Session Key =  7A D4 63 7D 0C B9 80 DC 70 BC 7A A5 D4 3C 2B 41

Expected MAC = BB 01 BD 09